# Screw Connectors

# 1500/ 4 DS

Terminal Block, 4 pole, Insulating Material: PA 6, Cross-

Section: 10.0 mm², Pin Spacing: 13.5 mm, Wire Protector (DS): yes, Fixing: Screw, Voltage Rating: 500 V, Current Rating: 57 A

# Order Info

Reference Number (ref. no.)	127104

Applications	Installation, Electrical Appliances, Transformers

PU	400

EAN	4028012106431

# Type

Type	Terminal Block

Number of poles	4 pole

Clamping Units per Pole	1

Colour	natural

Cross-Section	10.0 mm²

Connection Type	Screw

Wire Protector (DS)	yes

# Technical Data

Voltage Rating	500 V

Current Rating	57 A

Insulating Material	PA 6

Contact Material	Brass

Core Cross-Section min.	0.5 mm²

Core Cross-Section max.	6.0 mm²

Flammability Rating	V-2

max. Ambient Temperature Operation	85 °C

Type of Conductor Solid	yes

Type of Conductor Flexible	yes

Type of Conductor Stranded	yes

Stripping Length	8.0-8.5 mm

Assembly Contact Screw	0.8x4.0 /  1.2 Nm

Pin Spacing	13.5 mm

Fixing	Screw

Approbation EN60998	1500_EN60998_20070823_20160412_VDE_ENEC_40021887_web.p

Approbation UL	1500_2000_19890315_20170213_UL_E63492_1-08_Description_w

Approbation CSA	Schraub_Steck_Schraubenlos_SI_Klemmen_20160429_CSA_700765

# Accessories

ZEL 110	196420