date	12/06/2016

page 1 of 4

SERIES: SJ2-2591X-SMT │ DESCRIPTION: AUDIO JACK

FEATURES

- multiple switch configurations

- low profile design

- reflow solder compatible

- surface mount

# PART NUMBER KEY

SJ2-2591 X X

SMT-TR

-

Base Number

# Conductors: 21 = 2

3 = 3

Switch Options:

A = no switch

B = tip switch

C = ring switch

D = tip & ring switch

Package Style:

SMT = surface mount

Packaging:

TR = Tape & Reel

Note:	1. 2 conductor option only available with tip switch

| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| parameter | conditions/description | min | typ | max | units |
| rated input voltage |  |  | 12 |  | Vdc |
| rated input current |  |  |  | 1 | A |

| contact resistance2 | between terminal and mating plug between terminal in a closed circuit |  |  | 50 30 | mΩ mΩ |
| --- | --- | --- | --- | --- | --- |
| insulation resistance | at 500 Vdc | 100 |  |  | MΩ |
| voltage withstand | for 1 minute |  |  | 500 | Vac |
| insertion/withdrawal force |  | 0.3 |  | 3 | kg |
| operating temperature |  | -25 |  | 85 | °C |
| life | at a rate of 24 cycles/minute |  | 5,000 |  | cycles |
| flammability rating | UL94V-0 |  |  |  |  |
| RoHS | 2011/65/EU |  |  |  |  |

Note:	2. When measured at a current of less than 100 mA/1 kHz

3. All specifications measured at 10~35°C, humidity at 45~85%, under standard atmospheric pressure, unless otherwise noted.

# MECHANICAL DRAWING

|  | MATERIAL | PLATING |
| --- | --- | --- |
| terminal 1 | copper alloy | silver |
| terminal 2 | copper alloy | silver |
| terminal 3 | copper alloy | silver |
| terminal 10 | brass | silver |
| terminal 11 | brass | silver |
| plastic | PA10T |  |

units: mm tolerance:

X.X ±0.2 mm

X.XX ±0.10 mm

X.XXX ±0.050 mm

PCB: ±0.05 mm

Recommended PCB Layout Top View

| Model No. | SJ2-25912B-SMT | SJ2-25913A-SMT | SJ2-25913B-SMT | SJ2-25913C-SMT | SJ2-25913D-SMT |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | 1 | 1 | 1 |
| Schematic | 10 2 | 3 2 | 3 10 2 | 3 11 2 | 3 11 10 2 |
| PIN |  |  |  |  |  |
| 1 | sleeve | sleeve | sleeve | sleeve | sleeve |
| 2 | tip | tip | tip | tip | tip |
| 3 | NP | ring | ring | ring | ring |
| 10 | tip switch | NP | tip switch | NP | tip switch |
| 11 | NP | NP | NP | ring switch | ring switch |

1	3	2

11

Ø2.5 MATING PLUG

# SOLDERABILITY

parameter	conditions/description	min	typ	max	units

reflow soldering	see reflow profile	255	260	265	°C

270

260

250

240

Temperature (°C)

217

180

150

120

50

430

380

340

250

140

60

70

60	120	180	240	300	360	420	480

Time (sec)

# PACKAGING

units: mm

Reel Size: Ø330 mm

A: Empty 160 mm min. B: Empty 100 mm min. C: Cover 400 mm min.

Reel QTY: 1,000 pcs per reel

# REVISION HISTORY

| rev. | description | date |
| --- | --- | --- |
| 1.0 | initial release | 12/06/2016 |

The revision history provided is for informational purposes only and is believed to be accurate.

## Headquarters

20050 SW 112th Ave.

Tualatin, OR 97062

## 800.275.4899

Fax 503.612.2383

cui.com techsupport@cui.com

CUI offers a one (1) year limited warranty. Complete warranty information is listed on our website.

CUI reserves the right to make changes to the product at any time without notice. Information provided by CUI is believed to be accurate and reliable. However, no responsibility is assumed by CUI for its use, nor for any infringements of patents or other rights of third parties which may result from its use.

CUI products are not authorized or warranted for use as critical components in equipment that requires an extremely high level of reliability. A critical component is any component of a life support device or system whose failure to perform can be reasonably expected to cause the failure of the life support device or system, or to affect its safety or effectiveness.